Лексико-грамматический  тест по тексту «Electric current»

для студентов физического факультета,

изучающих английский язык 1,5 года

Составитель: Лозовская Т.В.

ELECTRIC CURRENT

If two metal spheres on insulating supports are charged with unlike electricity (say, sphere A positively and sphere В negatively) and connected by a metal conductor, electrons will flow from B, where they are in excess, to A where they are lacking. The flow of electrons in a conductor is called an electric current.

If sphere A is continuously charged with positive electricity and sphere В with negative electricity, there will be a continuous flow of electric current in the conductor. While in motion along a conductor, electrons collide with other electrons, atoms or molecules. As a result of these impacts, energy is released in the form of heat, and the motion of electrons along the conductor is impeded. This opposition to the motion of electrons along the conductor is known as the electrical resistance of the conductor.

There are several factors that affect resistance.

Resistance varies with the atomic structure or nature of the conducting material. Good conducting materials such as silver, copper, and aluminium have low resistance. Cast iron and nichrome (an alloy of iron, nickel, and copper) are examples of poorer conducting materials.

The resistance of most metals varies directly with temperature. The resistance of metals increases with increasing temperature, while that of liquids and carbon decreases. There are several metals, however, such as manganin, constantan, nickeline, etc., whose resistance remains practically unaffected by temperature rise.

The resistance of a conductor increases in direct proportion with its length. That is, temperature being constant, the resistance will be doubled if the length of the conductor is doubled.

The resistance of the conductor varies inversely with ts cross-section area.

The unit of electrical resistance is the ohm. The resistance in ohms of a conductor 1 metre long and 1 mm2 in cross-section is called resistivity and is designated by the Greek letter p (rho).

The resistance of a conductor can be found from the equation

where

R=plS

R — resistance of the conductor in ohms;

p — resistivity of the conductor, ohm-mm2-m;

l — length of the conductor in metres;

S— cross-section area of the conductor in mm2.

I. Answer the following questions.

1. What is called an electric current?

2. What is resistance?

3. What factors affect electrical resistance?

4. What is resistivity?

5. How can the resistance of a conductor be determined if its length, material and cross-section are known?

6. Define ohm.

7. What materials have low resistance?

8. Does the resistance of metals increase with increasing temperature?

9. Is the resistance of manganin affected by temperature rise?

II. Find the equivalents.

Simple, to term, to support, to decrease, unlike charges, lack, in terms of, except, to repel, to support, to increase, similar, as regards

За исключением, поддерживать, увеличивать, отталкивать, что касается, недостаток, называть, простой, на основании, уменьшать, разноименные заряды, поддерживать, похожий

III. Translate the following words.

Final, coefficient, conductor, metal, opposition, to separate

IV. Fill in the blanks with a suitable word. Use the correct form.

to repel, to attract, to affect, to increase, to decrease, to depend on, to contain, to vary, to consist of, an excess, a lack.

1. Various factors ... the resistance of a conductor. 2. The resistance of metals ... with a rise in temperature. 3. The resistance of carbon, semiconductors ... as their temperature is raised. 4. How does the resistance of a conductor ... with its length? 5. Objects that are similarly charged ... each other; those with unlike charges ... each other. 6. All matter ... both positive and negative charges. 7. An ordinary uncharged atom ... the same number of protons and electrons. 8. Resistivity ... the material composing the wire and its temperature. 9. The rubber rod becomes negatively charged due to ... of electrons and the fur becomes positively charged due to ... of electrons.

V. Give English equivalents for the following.

Постоянный ток, единица величины тока, направление, проводить, проводник, проводимость, сталкиваться, столкновение, количество электричества, сечение проводника, сопротивление, температурный коэффициент сопротивления, электрическое поле, магнитное папе, амперметр, повышать, понижать.